# Mechanical Properties of Metals

Often materials are subject to external force when they are used. Mechanical Engineers calculate those forces and material scientists how materials deform or break as a function of force, time, temperature, and other conditions. Materials scientists learn about these mechanical properties by testing materials.

Some of the important mechanical properties of a metals are Brittleness, Creep, Ductility, Elasticity, Fatigue, Hardness, Malleability, Plasticity, Resilience, Stiffness, Toughness, Yield strength. Above mechanical properties of metals are explained below in brief.

##### Brittleness:

Tendency of a material to fracture or fail upon the application of a relatively small amount of force, impact or shock.

##### Creep:

When a metal is subjected to a constant force at high temperature below its yield point, for a prolonged period of time, it undergoes a permanent deformation.

##### Ductility:

Ductility is the property by which a metal can be drawn into thin wires. It is determined by percentage elongation and percentage reduction in area of a metal.

##### Elasticity:

Elasticity is the tendency of solid materials to return to their original shape after being deformed.

##### Fatigue:

Fatigue is the material weakening or the breakdown of material subjected to stress, especially a repeated series of stresses.

##### Hardness:

Hardness is the ability of a material to resist permanent change of shape caused by an external force.

##### Malleability:

Malleability is the property by which a metal can be rolled into thin sheets.

##### Plasticity:

Plasticity is the property by which a metal retains its deformation permanently, when the external force applied on it is released.

##### Resilience:

Resilience is the ability of a metal to absorb energy and resist soft and impact load.

##### Stiffness:

When an external force is applied on a metal, it develops an internal resistance. The internal resistance developed per unit area is called stress. Stiffness is the ability of a metal to resist deformation under stress.

##### Toughness:

When a huge external force is applied on a metal, the metal will experience fracture. Toughness is the ability of a metal to resist fracture.

##### Yield strength:

The ability of a metal to bear gradual progressive force without permanent deformation.