REDTA Rail Project

This remit is in two parts. Part one is compulsory. Part two, the apprentice selects one item.

- PART ONE

This remit is to undertake a feasibility study for the construction of a new station. The station is to be built on an existing railway line, which is currently not electrified. The new station is to be built at the same time as the line is to be electrified with 25kV overhead line electrification (OLE).

The output from the study is a report and sketches/drawings as appropriate. The study shall consider options, which are to be assessed and from which a preferred option is selected. In addition, the study is to include an outline programme to undertake full design and evaluation.

## Station Requirements

- Disabled access to all platforms

- Station to include (plus associated facilities to support)

  - Waiting lounge

  - Ticket office

  - Small retail vending area (for light snacks and drinks)

  - Associated facilities (toilets, storage, office etc.)

- Security and communications systems: CCTV, ticketing, customer information systems, telephone system

- Building services; heating, lighting, small power, water, drainage

- Road access from the main road with lighting

- Car parking for 50 vehicles with lighting and CCTV. A form of car parking payment system is required.

- Options to consider

- In developing your feasibility study, the following three options shall be considered;

  - Single island platform with access from east side

  - Twin platforms with access from east side only

  - Twin platforms with access from both east and west side

Your study shall consider the impact of these options on the station requirements above.

- Technical Specification

- All buildings and platforms shall conform to Network Rail Standards

- The line-speed through the station is 90mph

- The station is on a level gradient

- Crossover speed is 25mph

- The crossover has electric points heating.

- Platform length is to be 180m

- Rolling stock, 8-car unit, each carriage 20m long (overall train length 160m)

- Driver only operation is not required

- Train frequency 4 per hour in each direction

- First train to stop in either direction 06.00. Last train to stop in either direction 23.00.

- No freight services run on this line.

- No trains run on Christmas Day or Boxing Day

- Possessions may be available over the Easter period, subject to agreement with the Train Operating Company

- Typical passenger entry/exit numbers 1500 per day

- OLE is to be type Mk3b, classic arrangement. The platform is mid span of an OLE tension length (1970m), so no need for overlaps or signal exclusion zones. The OLE design is to consider the OLE from the re-located crossover through the station and under the bridge, for a distance of one span either side. There is no need to make any separate provision for an electrical supply to the OLE

- Bridge soffit height is 4.8m

- There currently exists a standard three aspect signalling system with track circuits fed back to a signalling control centre. There is no plan to change this.

- Assume that the land is owned by Network Rail, with the exception of the highway and road overbridge. The land is generally flat, with no embankments or cuttings.

- Existing telecommunications system cables run through the site

- The nearest water supply and mains drainage/sewer runs along the main road.

- The nearest electricity supply (belongs to the electricity supply authority, not Network Rail) runs along the main road from the substation marked.

- The output from the study

The output from the study shall be a report that considers the merit of each of the three options, from which you select your preferred option. For each option you shall discuss their advantages and disadvantages and a clear reason for your preferred option.

For your preferred option, you shall explain your proposed solution and include answers to the following questions;

- What planning requirements are necessary?

- What environmental considerations are necessary?

- What existing technical information is required?

- What record information is required?

- What survey information is required?

- What are the key interfaces?

- What are the key project risks?

- What standards are applicable?

For each railway discipline, you shall include a brief description of the scope of their work, the impact on other disciplines and in particular;

- Track: What changes to the track are required and why? In particular where is the cross over to be moved?

- Civil and structural: what building work is required for the station buildings, platform, footbridge, car park and access roads? What is the typical size of the buildings proposed?

- OLE: What OLE structures are required and how will that impact the platform and the bridge?

- Electrical, mechanical and building services: What services are required and where? If a lift is proposed, what sort?

- Telecommunications: what services are required and where. What electrical supplies are needed?

- Signalling: what is the impact on the signalling system? Will any new signals be required?

- Report Structure

The report shall be well-structured (your company standard report template can be used) and should at least contain the following headings;

- Introduction. This should describe the project, the project objectives and outputs.

- Existing Infrastructure. A brief overview of what currently is at that location and where existing systems are located

- Proposed Options. Discuss the proposed options and their merits.

- Preferred Option. Describe the preferred option including answers to the questions in Section 4.0. Give an overview of what you are proposing with a brief description of each asset. Identify the work required for each discipline.

- Assumptions

- Interface issues

- Project risks

- Programme to undertake a full design.

- Drawings. To include as a minimum a general layout plan showing the station footprint, platforms, track, signals and OLE

- PART TWO

The objective of Part Two of this project is to undertake an outline design/detailed report covering one of the specialist areas listed below. The output will be a report and drawings giving more detail of one of the discipline requirements. Again your report should be clearly written and follow a structure that you can choose. Select one specialist area and provide a report as required. For each topic the following must be produced;

- Design log

- Assumptions log

- Risk register

- Civil Engineering;

- What are the options for constructing the station, what are their merits and submit your preferred proposal

- What are the key interfaces that need addressing

- Produce an outline plan of the station footprint to include access road and car park

- What are the specific design risks

- Produce an estimate in hours for undertaking the design study and an outline programme

- Track Engineering:

- The existing cross over will have to be moved to accommodate the station platforms. What is the design process for doing this?

- What are the key interfaces

- What information is required

- What are the specific design risks

- Produce an outline track plan showing the track changes.

- Produce an estimate in hours for undertaking the design study and an outline programme

- Signalling Engineering

- What are the signalling requirements for this work? What is the design process for doing this?

- What are the key interfaces

- What information is required

- What are the specific design risks

- Produce an outline scheme plan.

- Produce an estimate in hours for undertaking the design study and an outline programme

- Overhead Electrification (OLE)

- What are the options for electrifying the area through the station and including the cross over (consider only 500m either side of the station as a stand-alone design)

- What are the key interfaces

- What information is required

- What are the specific design risks

- Produce an outline OLE plan.

- Produce an estimate in hours for undertaking the design study and an outline programme

- Telecoms Engineering

- What telecoms requirements are required for this site?

- What are the key interfaces

- What information is required

- What are the specific design risks

- Produce a telecoms schematic.

- Produce an estimate in hours for undertaking the design study and an outline programme

- Electrical Engineering

- What electrical load will the station requirements generate and how is this best distributed?

- What are the key interfaces

- What information is required

- What are the specific design risks

- Produce an electrical schematic.

- Produce an estimate in hours for undertaking the design study and an outline programme

- Survey

- What specific survey requirements are required for this work?

- What safety paperwork needs to be put in place and what track access is required?

- What are the key interfaces

- What information is required

- What are the specific survey risks

- Produce an estimate in hours for undertaking the surveys and an outline programme