Design a common emitter amplifier using a 2n3055 transistor.  The amplifier is to provide audio frequency amplification for an audio amplifier and will be powered from a car battery.  The amplifier will use negative feedback (NFB)

You will need to:

Find out the voltage range for a car battery

Find a datasheet for the 2n3055

State your assumptions and rules of thumb you will use to create the amplifier

Draw the circuit diagram

Show calculations for all resistor values

Simulate the amplifier on MultiSim and show how it amplifies a 1kHz 2V AC signal