SECONDARY CELL CHEMISTRY
The Difference between primary and secondary cells are: the secondary cell can be recharged and the electrodes are made of different materials. The secondary cell shown in figure 2-3uses sponge lead as the cathode and lead peroxide as the anode. This is the lead-acid type cell and will be used to explain the general chemistry of the secondary cell. Later in the chapter when other types of secondary cells are discussed, you will see that the materials which make up the parts of a cell are different, but that the chemical action is essentially the same.

[image: image1.png]AP,
SoLuTIoN — ] I SOLUTION Ll

SULFURIC ACID
1,50,
AND WATER 0

suLEuRE Ac

i
P o
i

Ll

water

sPoNGE Ledp

Leap. PERONDE

P PEO,

(4) CHARGED (B) DISCHARGING
DECREASING  DECREASNG
SPONGE LEAD  LEAD PEROKIDE
WCREASNG,  NCREASNO

LEAD SULFATE  LEAD SULFATE

O PoePhsGy | PaogPus0,

soreron soumon
=
i
suirym aco sutie R
=]
waren f—
o, g
(D} CHARGING {C) DISCHARGED
e -
Mhesito lObiowe  sowiow Lo o
SEtehe.  beatan 2 it
e Sbdumwe (o ioo s
EREs BT Hee

wowce a0 5 Leao penorpe ol Lekn suusate



The Figure view A shows a lead-acid secondary cell that is fully charged. The cathode is pure sponge lead, the anode is pure lead peroxide, and the electrolyte is a mixture of sulfuric acid and water.

The Figure view B shows the secondary cell discharging. A load is connected between the cathode and anode; current flows negative to positive as shown. This current flow creates the same process as was explained for the primary cell with the following exceptions.

In the primary cell the zinc cathode was eaten away by the sulfuric acid. In the secondary cell the sponge-like construction of the cathode retains the lead sulphate formed by the chemical action of the sulfuric acid and the lead. In the primary cell the carbon anode was not chemically acted upon by the sulfuric acid. In the secondary cell the lead peroxide anode is chemically changed to lead sulphate by the sulfuric acid.

When the cell is fully discharged it will be as shown in Figure view C. The anode and cathode retain some lead peroxide and sponge lead but the amounts of lead sulphate in each is maximum. The electrolyte has a minimum amount of sulfuric acid. With this condition no further chemical action can take place within the cell.

As you may know, the secondary cell can be recharged. Recharging is the process of reversing the chemical action that occurs as the cell discharges. To recharge the cell, a voltage source, such as a generator, is connected as shown in figure view D. The negative terminal of the voltage source is connected to the cathode of the cell and the positive terminal of the voltage source is connected to the anode of the cell. With this arrangement the lead sulphate is chemically changed back to sponge lead in the cathode, lead peroxide in the anode, and sulfuric acid in the electrolyte. After all the lead sulphate is chemically changed, the cell is fully charged as shown in figure view A. Once the cell has been charged, the discharge-charge cycle may be repeated. 
